# FibreLink ONT-100 quick guide

The FibreLink ONT-100 is the optical network terminal that NetCo installs at fiber customers on a GPON line. This guide is for the support team.

# Factory reset

To restore the ONT-100 to factory settings, hold the recessed reset button for 12 seconds with a paperclip. The ONT restarts within three minutes and must then be provisioned again by the access team. Reset the ONT only when the access team asks for it.

# Optical power levels

| Receive power | Meaning |
| --- | --- |
| -8 to -27 dBm | Normal receive power |
| Below -27 dBm | Signal too weak: check the fiber patch cable, then book an engineer |
| Above -8 dBm | Signal too strong (overload) |

# Handling fiber connectors

Never look into the end of a fiber or an ONT port. Keep the dust caps on unused connectors and do not bend a patch cable tighter than 30 mm.